# Project PRD: SWMM Hydraulic Model Automation (Codex Version)

## 1. Overview

This project aims to automate the construction of a SWMM hydraulic model from marked‑up plan PDFs and a rational‑method Excel file. The goal is to build a reusable Codex workflow that ingests data, validates it, creates a fully functioning SWMM model (hydraulics + simplified hydrology), and iterates through verification loops. We assume homogeneous subcatchments and will generate them automatically based on the rational‑method areas and heuristic widths.

## 2. Objectives & Scope

- Extract data from the Excel file into structured tables (nodes, links, inflows, and subcatchments).

- Validate connectivity and data quality using RALPH loops (Reflect‑Act‑Learn‑Plan‑Hone), flagging issues and asking clarifying questions via an Ask‑User tool.

- Generate a SWMM .inp file including junctions, conduits, cross sections, subcatchments, inflows, and default catchment parameters.

- Run and verify the model (continuity checks, stable hydraulics) and iterate as needed.

- Provide outputs and reports (QA logs, summary statistics, diagrams) to the user.

- Define a plan map for sub‑agents to handle different stages of the workflow.

### Out of Scope

- Exact catchment geometry from GIS; we will approximate subcatchments based on rational method data.

- Detailed hydrological calibration beyond using homogeneous basins and rational inflows.

- Manual placement of coordinates on the SWMM backdrop (can be added later as an enhancement).

## 3. Assumptions

- The Excel file contains a complete set of nodes, links, pipe sizes, slopes, lengths, and rational method data.

- Inlet nodes in the PDFs correspond to node IDs in the Excel file.

- Catchment areas listed in the Excel file will be treated as the total subcatchment area for each corresponding node.

- Width heuristics: Width = 2 × Area / TcLength (since width is often approximated as Area divided by half of the flow length).

- All subcatchments use default infiltration and percent impervious parameters unless specified by the user later.

## 4. Functional Requirements

- Data Extraction Module

- Parse the .xlsm file to extract:

  - Node IDs, rim elevations, invert elevations (in/out), node types (junction/manhole/outfall).

  - Link information: upstream node, downstream node, diameter/shape, length, slope, Manning’s n.

  - Rational method results: Tc, C, Area, intensity (i), Q.

- Output structured tables (e.g., CSV or DataFrames) for nodes, links, inflows, and rational data.

- QA & Validation Module

- Build a directed graph from links to detect orphan nodes, duplicate IDs, cycles, negative slopes, and missing inverts.

- Use RALPH loops to verify and repair:

  - Reflect: Summarize detected issues.

  - Act: Apply programmatic fixes where possible (e.g., infer downstream node for a split, fill missing invert with rim minus pipe diameter).

  - Learn/Plan: Update QA logic based on outcomes; adjust heuristics.

  - Hone: Record improvements or unresolved issues.

- If ambiguous data cannot be resolved automatically, call the Ask‑User tool: present context and a clear question to the user (e.g., “Node A103 has no downstream pipe; is it an outfall?”).

- SWMM Model Generation Module

- Use the swmm_api or similar library to build a SWMM .inp file from the tables:

  - [JUNCTIONS] / [OUTFALLS]: insert node coordinates if available (dummy values otherwise), rim/invert elevations.

  - [CONDUITS]: assign link parameters (US/DS nodes, lengths, slopes, diameters).

  - [XSECTIONS]: define circular pipes or other shapes based on diameter.

  - [INFLOWS]: assign rational Q as external inflows at nodes.

  - [SUBCATCHMENTS]: for each node, create a subcatchment with Area (from Excel), Outlet (that node), Width (heuristic), Slope (use pipe slope or global default), %Imperv (default), and relevant parameters in [SUBAREAS] and [INFILTRATION] sections.

- Write the .inp file to the workspace.

- Simulation & Verification Module

- Use pyswmm to run the model and check for:

  - Hydraulic continuity errors.

  - Stability (no surcharging anomalies or errors).

  - Flow conservation (sum of inflows ≈ sum of outflows at outfalls).

- Use RALPH loops to reflect on results, adjust parameters (e.g., Manning’s n, widths), and iterate.

- User Interaction & QA Logging

- Implement the Ask‑User tool to prompt the user when data is missing or decisions are ambiguous.

- Provide the user with interim QA reports (lists of flagged issues, summary statistics, suggestions for improvements).

- Document all decisions and resolutions for transparency.

- Plan Map & Sub‑Agent Roles

- Extractor Agent: Reads the Excel, populates tables.

- Validator Agent: Performs QA checks using RALPH loops and consults the Ask‑User tool.

- Model Builder Agent: Generates the SWMM .inp file (nodes, links, inflows, subcatchments).

- Simulation Agent: Runs the model, collects results, and triggers further RALPH loops.

- Coordinator Agent: Orchestrates sub‑agents, manages dependencies, and logs progress.

- User‑Interaction Agent: Handles the Ask‑User tool, presenting questions and capturing responses.

## 5. Non‑functional Requirements

- Repeatability: The workflow must run end‑to‑end via script without manual intervention (except via Ask‑User prompts).

- Scalability: Data extraction and model generation should handle networks with hundreds of nodes and links.

- Modularity: Agents must be modular; improvements to one module should not require rewriting others.

- Transparency: All assumptions, data transformations, and QA results should be logged for auditing.

## 6. Acceptance Criteria

- Data extraction creates four tables: nodes.csv, links.csv, rational_data.csv, and subcatchment_defaults.csv.

- QA & Validation identifies no unresolved orphan nodes, negative slopes (unless justified), or missing crucial data.

- The SWMM .inp file runs without errors and produces a valid report.

- The sum of inflows equals the sum of outflow at the outfall within ±1%.

- The user can respond to prompts from the Ask‑User tool to resolve ambiguous information.

- An optional QA report summarizing anomalies and suggested corrections is produced.

## 7. Plan Map & Timeline

| Phase | Description | Agents Involved | Deliverables | Duration (est.) |
| --- | --- | --- | --- | --- |
| 1. Initialization | Set up project repo, define PRD and plan map. | Coordinator | prd.md, plan map document | 1 day |
| 2. Data Extraction | Parse Excel, produce tables. | Extractor | nodes.csv, links.csv, rational_data.csv | 1–2 days |
| 3. QA & Validation | Build graph, run RALPH loops, ask user clarifications. | Validator, User‑Interaction | QA reports, resolved issues log | 2–3 days |
| 4. Model Generation | Create .inp file with nodes, links, inflows, and subcatchments. | Model Builder | model.inp | 1 day |
| 5. Simulation & Iteration | Run model, check stability, adjust parameters. | Simulation | results summary, revised model | 2 days |
| 6. Finalization | Compile final model, deliver QA logs and instructions for future updates. | Coordinator | final .inp, logs, user guide | 1 day |

Note: Time estimates assume moderate complexity; adjust based on network size.

## 8. Future Enhancements

- Geospatial Integration: If CAD/GIS data becomes available, integrate coordinates and catchment polygons.

- Hydrologic Calibration: Introduce dynamic hyetographs, infiltration models, and calibration tools.

- Visualization: Generate plan diagrams and HGL plots automatically.

- Continuous Integration: Deploy automated tests for new data updates.

## 9. Risks & Mitigation

- Missing or inconsistent data → Mitigation: Use RALPH loops and Ask‑User prompts to clarify.

- Unstable hydraulics → Mitigation: Adjust time step, conduit roughness, and width heuristics iteratively.

- Tooling limitations → Mitigation: Validate the selected Python libraries (swmm_api, pyswmm) early and pivot if necessary.

- Scope creep → Mitigation: Stay focused on Path A with homogeneous basins; schedule Path B improvements separately.